LAB MANUAL

EDA Workshop

(ECW 205B)

PREPARED BY: MEENAKSHI GUPTA

FOR

IV SEMESTER CSE

Department of Electronics & Communication Engg.

MANAV RACHNA UNIVERSITY

SECTOR-43, FAR I D A B A D – 121001

LIST OF EXPERIMENTS

- Simulate and study V-I characteristics of a Diode using PSPICE windows.

- Simulate and study Half-wave and Full-wave Rectifier using PSPICE windows

- Simulate and study Diode Clipper and Clamper circuit using PSPICE windows.

- Simulate and study V-I characteristics of a NPN-BJT using PSPICE windows.

- Simulate and study Darlington pair amplifier circuit using PSPICE windows and   determine quiescent condition.

- Simulate and study transient & frequency response of a BJT amplifier in common- emitter configuration using PSPICE windows.

- Simulate and study active low-pass, high-pass & band-pass filter using PSPICE     windows.

- Simulate and study Integrator using PSPICE windows.

- Simulate and study Differentiator using PSPICE windows.

- Simulate and study basic AND, OR, NOT, NOR, NAND, EX-OR gates using PSPICE     windows.

# EXPERIMENT NO 1

AIM: Simulate and study V-I characteristics of a Diode using PSPICE windows.

CIRCUIT DIAGRAM:

PROGRAM:

vd 1 0 dc 1v

d1 1 2 d1n914;

vx 2 0 dc 0v;

.model D1N914 D(Is=168.1E-21 N=1 Rs=.1 Ikf=0 Xti=3 Eg=1.11 Cjo=4p M=.3333

+	Vj=.75 Fc=.5 Isr=100p Nr=2 Bv=100 Ibv=100u Tt=11.54n)

.dc vd 0 1v 0.01v;

.plot dc I(vx);

.probe;

.tran 0us 100us;

.end;

RESULT:

CONCLUSION: The forward biased silicon diode in an electronic system under dc condition has a drop of 0.7v across it in conduction state at any value of diode current.

EXPERIMENT NO 2

AIM: Simulate and study Half-wave and Full-wave Rectifier using PSPICE windows.

CIRCUIT DIAGRAM:

## PROGRAM:

*HALF WAVE rectifier VIN 2 0 sin(0 220V 50HZ)

RL 5 0 500

RS 2 1 10

L1 1 0 2000uH

L2 3 0 20uH

K1 L1 L2 0.99999 D1 3 5 D1N4009

.model D1N4009	D(Is=544.7E-21 N=1 Rs=.1 Ikf=0 Xti=3 Eg=1.11 Cjo=4p M=.3333

+	Vj=.75 Fc=.5 Isr=30.77n Nr=2 Bv=25 Ibv=100u Tt=2.885n)

.tran 0.2m 200m

.plot tran v(3), v(5)

.probe

.end

*FULL WAVE rectifier Vin 2 0 sin(0 230V 50HZ)

RL 5 4 1000

RS 2 1 10

L1 1 0 2000

L2 3 4 10

L3 4 0 10

K1 L1 L2 L3 0.99 D1 0 5 D1N4009

D2 3 5 D1N4009

.model D1N4009

D(Is=544.7E-21 N=1 Rs=.1 Ikf=0 Xti=3 Eg=1.11 Cjo=4p M=.3333

+	Vj=.75 Fc=.5 Isr=30.77n Nr=2 Bv=25 Ibv=100u Tt=2.885n)

.tran 0.2ms 200ms

.probe

.end

RESULT:

CONCLUSION: We are able to visualise the expected output of Half-wave and Full-wave Rectifier circuit as given in theory.

EXPERIMENT NO 3

AIM: Simulate and study Diode Clipper and Clamper circuit using PSPICE windows.

CIRCUIT DIAGRAM:

Clipper Circuit-

Clamper Circuit-

PROGRAM:

*CLIPPER CIRCUIT

V_VAC 1 0 sin(0 5v 30khz);

r1 1 2 0.22k;

d1 2 3 D1N914;

d2 4 2 D1N914;

vx1 3 0 dc 1v;

vx2 0 4 dc 1.5v;

.model D1N914 D(Is=168.1E-21 N=1 Rs=.1 Ikf=0 Xti=3 Eg=1.11 Cjo=4p M=.3333

+	Vj=.75 Fc=.5 Isr=100p Nr=2 Bv=100 Ibv=100u Tt=11.54n)

.probe;

.tran 0us 100us;

.end;

*CLAMPER CIRCUIT

V1 1 0 sin(0 5v 30khz);

c1 1 2 0.01u;

d1 3 2 D1N914;

r1 2 0 560k;

v2 3 0 1v;

.model D1N914 D(Is=168.1E-21 N=1 Rs=.1 Ikf=0 Xti=3 Eg=1.11 Cjo=4p M=.3333

+	Vj=.75 Fc=.5 Isr=100p Nr=2 Bv=100 Ibv=100u Tt=11.54n)

.probe;

.tran 0us 100us;

.end;

RESULT:

CONCLUSION: We are able to visualise the expected output of Clipper Circuit and Clamper Circuit as given in theory.

EXPERIMENT NO 4

AIM: Simulate and study V-I characteristics of a NPN-BJT using PSPICE windows.

1

Q1 Q2N2222A

IB

1mAdc

CIRCUIT DIAGRAM:

VCE

12Vdc

## PROGRAM:

* NPN-BJT CHARACTERISTICS IB	0 1 DC 1MA

VCE 2 0 DC 12V

Q1	2  1  0	Q2N2222A	*Tools – Pspice - Library - Bipolar

.MODEL Q2N2222A NPN (IS=2.105E-16 BF=173 VA=83.3V CJE=29.6PF CJC=19.4PF

+	TF=489.88PS TR=4.9NS)

.DC VCE 0 10V 0.02V IB 0 1MA 200UA

.PROBE

.OP

.END

RESULT:

CONCLUSION: We are able to visualise the expected output of NPN-BJT circuit as given in theory.

EXPERIMENT NO 5

AIM: Simulate and study Darlington pair amplifier circuit using PSPICE windows and determine quiescent condition.

CIRCUIT DIAGRAM:

PROGRAM:

VCC 2 0 DC 12V

VIN 1 0 DC 5V

Q1  2 1 3 Q2N2484	* BJTs with model (Q2N2484) Q2 2 3 4 Q2N2484

RB 2 1 47k

RE 4 0 4.7K

*	Model Q2N2484 for NPN BJTs

.model Q2N2484	NPN(Is=5.911f Xti=3 Eg=1.11 Vaf=62.37 Bf=697.1 Ne=1.342

+	Ise=5.911f Ikf=13.93m Xtb=1.5 Br=1.297 Nc=2 Isc=0 Ikr=0 Rc=1.61

+	Cjc=4.017p Mjc=.3174 Vjc=.75 Fc=.5 Cje=4.973p Mje=.4146 Vje=.75

+	Tr=4.687n Tf=820.4p Itf=.35 Vtf=4 Xtf=7 Rb=10)

* Transfer function analysis to calculate dc gain, input resistance and output resistance

.TF V(4) VIN

.END

RESULT:

**** 11/11/13 09:10:06 ******* PSpice Lite (April 2011) ******* ID# 10813 ****

**** Darlington Pair

****	SMALL SIGNAL BIAS SOLUTION	TEMPERATURE = 27.000 DEG C

***************************************************************************           NODE VOLTAGE	NODE VOLTAGE	NODE VOLTAGE	NODE VOLTAGE

(	1)	5.0000 (	2) 12.0000 (	3)	4.3560 (	4)	3.5909

VOLTAGE SOURCE CURRENTS NAME	CURRENT

VCC	-9.129E-04

VIN	1.489E-04

TOTAL POWER DISSIPATION 1.02E-02 WATTS

****	SMALL-SIGNAL CHARACTERISTICS V(4)/VIN = 9.851E-01

INPUT RESISTANCE AT VIN = 4.696E+04 OUTPUT RESISTANCE AT V(4) = 6.677E+01

JOB CONCLUDED

**** 11/11/13 09:10:06 ******* PSpice Lite (April 2011) ******* ID# 10813 ****

**** Darlington Pair

****	JOB STATISTICS SUMMARY

***************************************************************************            Total job time (using Solver 1) =	0.00

EXPERIMENT NO 6

AIM: Simulate and study transient & frequency response of a BJT amplifier in common- emitter configuration using PSPICE windows.

CIRCUIT DIAGRAM:

RC

R1

47k

10k

C2

4

6

Rs

1 	 2

500

C1

10uF

3

Q1

10uF

Q2N2907

5

VIN

R2

5k

RL

20k

RE

2k

CE

10uF

Vcc 15Vdc

VOFF = 0 VAMPL = 10mV FREQ = 1kHz

PROGRAM:

*

.AC DEC 10 1hz 10khz;

VIN 1 0 AC 10mv sin(0 10mv 1khz); VCC 0 7 DC 15v;

RS 1 2 500;

R1 7 3 47k;

R2 3 0 5k;

RC 7 4 10k;

RE 5 0 2k;

RL 6 0 20k;

C1 2 3 10uF;

C2 4 6 10uF;

CE 5 0 10uF;

Q1 4 3 5 0 Q2N2907;

.model Q2N2907	PNP(Is=650.6E-18 Xti=3 Eg=1.11 Vaf=115.7 Bf=231.7 Ne=1.829

+	Ise=54.81f Ikf=1.079 Xtb=1.5 Br=3.563 Nc=2 Isc=0 Ikr=0 Rc=.715

+	Cjc=14.76p Mjc=.5383 Vjc=.75 Fc=.5 Cje=19.82p Mje=.3357 Vje=.75

+	Tr=111.3n Tf=603.7p Itf=.65 Vtf=5 Xtf=1.7 Rb=10)

- National	pid=63	case=TO18

- 88-09-09 bam creation

.tran 50us 2ms;

.plot tran V(4) V(6) V(1);

.plot AC VM(6) VP(6);

.probe;

.end;

RESULT:

CONCLUSION: We are able to visualise the expected output of transient & frequency response of a BJT amplifier in common-emitter configuration as given in theory.

EXPERIMENT NO 7

AIM: Simulate and study active low-pass, high-pass & band-pass filter using PSPICE windows.

CIRCUIT DIAGRAM:

RG	RF

10k

100k

3 +

OUT

1

Vo

R1

2 -

UA741

8

V-

V+

Low-pass Filter

10k

Vin

4

C1

0.1uF

R4

10k

R2

20k C2

1n

R1

3 + 20k

R5

OUT

1

Vo

2

10k

V1

-

UA741

R3

10k

4

8

V-

V+

High-pass Filter

R4

10k	R2

R1

3 + 20k

-

2

V-

V1

20k C2

8

V+

1n

OUT 1

R5

10k

8

3    +

V+

OUT 1

### Band-pass Filter

Vo

4

UA741

R3

2 -

V-

UA741

4

10k	R6

100k

R7

100k

## PROGRAM:

*Low Pass Filter (BY UA 741) VIN 1 0 AC 1

RG 3 0 10K

RF 3 4 100K

R1 1 2 15.9K

C1 2 0 .01UF

XA1 2 3 7 4 6 UA741

*-----------------------------------------------------------------------------

- connections: non-inverting input

- | inverting input

- | | positive power supply

- | | | negative power supply

- | | | | output

*	| | | | |

.subckt uA741 1 2 3 4 5

* c1 11 12 8.661E-12

c2 6 7 30.00E-12

dc  5 53 dy

de  54 5 dy

dlp 90 91 dx

dln 92 90 dx

dp 4 3 dx

egnd 99 0 poly(2),(3,0),(4,0) 0 .5 .5

fb 7 99 poly(5) vb vc ve vlp vln 0 10.61E6 -1E3 1E3 10E6 -10E6 ga 6 0 11 12 188.5E-6

gcm 0 6 10 99 5.961E-9

iee 10 4 dc 15.16E-6

hlim 90 0 vlim 1K

q1 11 2 13 qx

q2 12 1 14 qx

r2 6 9 100.0E3

rc1 3 11 5.305E3

rc2 3 12 5.305E3

re1 13 10 1.836E3

re2 14 10 1.836E3

ree 10 99 13.19E6

ro1 8 5 50

ro2 7 99 100

rp 3 4 18.16E3

vb   9 0 dc 0

vc 3 53 dc 1

ve 54 4 dc 1

vlim 7 8 dc 0

vlp 91 0 dc 40

vln 0 92 dc 40

.model dx D(Is=800.0E-18 Rs=1)

.model dy D(Is=800.00E-18 Rs=1m Cjo=10p)

.model qx NPN(Is=800.0E-18 Bf=93.75)

.ends

.AC DEC 10 10HZ 100GHZ

.PLOT AC VM(4) VP(4)

.PROBE

.END

- HIGH PASS Filter (BY UA741) VIN 1 0 AC 1

RG 3 0 10K

RF 3 4 10K

C1 1 2 .01UF

R1 2 0 15.9K

XA1 2 3 7 4 6 UA741

*-----------------------------------------------------------------------------

- connections: non-inverting input

- | inverting input

- | | positive power supply

- | | | negative power supply

- | | | | output

*	| | | | |

.subckt uA741 1 2 3 4 5

*

c1 11 12 8.661E-12

c2 6 7 30.00E-12

dc  5 53 dy

de  54 5 dy

dlp 90 91 dx

dln 92 90 dx

dp 4 3 dx

egnd 99 0 poly(2),(3,0),(4,0) 0 .5 .5

fb 7 99 poly(5) vb vc ve vlp vln 0 10.61E6 -1E3 1E3 10E6 -10E6 ga 6 0 11 12 188.5E-6

gcm 0 6 10 99 5.961E-9

iee 10 4 dc 15.16E-6

hlim 90 0 vlim 1K

q1 11 2 13 qx

q2 12 1 14 qx

r2 6 9 100.0E3

rc1 3 11 5.305E3

rc2 3 12 5.305E3

re1 13 10 1.836E3

re2 14 10 1.836E3

ree 10 99 13.19E6

ro1 8 5 50

ro2 7 99 100

rp 3 4 18.16E3

vb   9 0 dc 0

vc 3 53 dc 1

ve 54 4 dc 1

vlim 7 8 dc 0

vlp 91 0 dc 40

vln 0 92 dc 40

.model dx D(Is=800.0E-18 Rs=1)

.model dy D(Is=800.00E-18 Rs=1m Cjo=10p)

.model qx NPN(Is=800.0E-18 Bf=93.75)

.ends

.AC DEC 10 10HZ 100KHZ

.PLOT AC VM(4) VP(4)

.PROBE

.END

*BAND PASS FILTER VIN 1 0 AC 1V

R1 1 2 5K

R2 3 4 1.5K

R3 2 0 265K

C1 2 4 0.01UF

C2 2 3 0.01UF

RL 4 0 15K

VCC 6 0 DC 12V

VEE 0 7 DC 12V

XA1 0 3 6 7 4 UA741

.subckt uA741 1 2 3 4 5

*

c1 11 12 8.661E-12

c2 6 7 30.00E-12

dc  5 53 dy

de  54 5 dy

dlp 90 91 dx

dln 92 90 dx

dp 4 3 dx

egnd 99 0 poly(2),(3,0),(4,0) 0 .5 .5

fb 7 99 poly(5) vb vc ve vlp vln 0 10.61E6 -1E3 1E3 10E6 -10E6 ga 6 0 11 12 188.5E-6

gcm 0 6 10 99 5.961E-9

iee 10 4 dc 15.16E-6

hlim 90 0 vlim 1K

q1 11 2 13 qx

q2 12 1 14 qx

r2 6 9 100.0E3

rc1 3 11 5.305E3

rc2 3 12 5.305E3

re1 13 10 1.836E3

re2 14 10 1.836E3

ree 10 99 13.19E6

ro1 8 5 50

ro2 7 99 100

rp 3 4 18.16E3

vb   9 0 dc 0

vc 3 53 dc 1

ve 54 4 dc 1

vlim 7 8 dc 0

vlp 91 0 dc 40

vln 0 92 dc 40

.model dx D(Is=800.0E-18 Rs=1)

.model dy D(Is=800.00E-18 Rs=1m Cjo=10p)

.model qx NPN(Is=800.0E-18 Bf=93.75)

.ends

.AC DEC 10 100HZ 1MEGHZ

.PLOT AC VM(4) VP(4)

.PROBE

.END

## RESULT:

*LOW PASS CIRCUIT

RESULT:

*HIGH PASS CIRCUIT

*BAND PASS CIRCUIT

CONCLUSION: We are able to visualise the expected output of active low-pass, high-pass & band-pass filter as given in theory.

EXPERIMENT NO 8

AIM: Simulate and study Integrator using PSPICE windows.

CIRCUIT DIAGRAM:	C1

R1

Vin

1	2

2.5k

0.1uF

Rf

4

8

1MEG

V+

3	+

OUT	1

output voltage

V-

3	2	-

4

Rx

2.5k

Rl

100k

## PROGRAM:

* INTEGRATOR

VIN 1 0 PWL(0 0 1NS -1V 1MS -1V 1.0001MS 1v 2ms 1v 2.0001ms -1V 3MS -1V 3.0001MS 1V 4MS 1V)

R1 1 3 2.5K

RF 3 6 1MEG

RX 2 0 2.5K

RL 6 0 100K

C1 3 6 0.1UF

XA1 2 3 7 4 6 UA741

*-----------------------------------------------------------------------------

- connections: non-inverting input

- | inverting input

- | | positive power supply

- | | | negative power supply

- | | | | output

*	| | | | |

.subckt uA741 1 2 3 4 5

*

c1 11 12 8.661E-12

c2 6 7 30.00E-12

dc  5 53 dy

de  54 5 dy

dlp 90 91 dx

dln 92 90 dx

dp 4 3 dx

egnd 99 0 poly(2),(3,0),(4,0) 0 .5 .5

fb 7 99 poly(5) vb vc ve vlp vln 0 10.61E6 -1E3 1E3 10E6 -10E6 ga 6 0 11 12 188.5E-6

gcm 0 6 10 99 5.961E-9

iee 10 4 dc 15.16E-6

hlim 90 0 vlim 1K

q1 11 2 13 qx

q2 12 1 14 qx

r2 6 9 100.0E3

rc1 3 11 5.305E3

rc2 3 12 5.305E3

re1 13 10 1.836E3

re2 14 10 1.836E3

ree 10 99 13.19E6

ro1 8 5 50

ro2 7 99 100

rp 3 4 18.16E3

vb   9 0 dc 0

vc 3 53 dc 1

ve 54 4 dc 1

vlim 7 8 dc 0

vlp 91 0 dc 40

vln 0 92 dc 40

.model dx D(Is=800.0E-18 Rs=1)

.model dy D(Is=800.00E-18 Rs=1m Cjo=10p)

.model qx NPN(Is=800.0E-18 Bf=93.75)

.ends

.TRAN 50US 4MS

.PROBE

.END

## RESULT:

CONCLUSION: We are able to visualise the expected output of Integrator circuit as given in theory.

EXPERIMENT NO 9

AIM: Simulate and study Differentiator using PSPICE windows.

CIRCUIT DIAGRAM:

R1	C1

100

Vin

1	2	3

0.4uF

Rf

4

10k

8

V+

3	+

OUT	1

V-

5	2	-

output voltage

4

Rx

10k

Rl

100k

## PROGRAM:

*DIFFERENTIATOR CIRCUIT

VIN 1 0 PWL(0 0 1MS 1 2MS 0 3MS 1 4MS 0)

R1 1 8 82

RF 3 6 1.5K

RX 2 0 1.5K

RL 6 0 10K

C1 8 3 0.1UF

CF 3 6 .005UF

XA1 2 3 7 4 6 UA741

*-----------------------------------------------------------------------------

- connections: non-inverting input

- | inverting input

- | | positive power supply

- | | | negative power supply

- | | | | output

*	| | | | |

.subckt uA741 1 2 3 4 5

*

c1 11 12 8.661E-12

c2 6 7 30.00E-12

dc  5 53 dy

de  54 5 dy

dlp 90 91 dx

dln 92 90 dx

dp 4 3 dx

egnd 99 0 poly(2),(3,0),(4,0) 0 .5 .5

fb 7 99 poly(5) vb vc ve vlp vln 0 10.61E6 -1E3 1E3 10E6 -10E6 ga 6 0 11 12 188.5E-6

gcm 0 6 10 99 5.961E-9

iee 10 4 dc 15.16E-6

hlim 90 0 vlim 1K

q1 11 2 13 qx

q2 12 1 14 qx

r2 6 9 100.0E3

rc1 3 11 5.305E3

rc2 3 12 5.305E3

re1 13 10 1.836E3

re2 14 10 1.836E3

ree 10 99 13.19E6

ro1 8 5 50

ro2 7 99 100

rp 3 4 18.16E3

vb   9 0 dc 0

vc 3 53 dc 1

ve 54 4 dc 1

vlim 7 8 dc 0

vlp 91 0 dc 40

vln 0 92 dc 40

.model dx D(Is=800.0E-18 Rs=1)

.model dy D(Is=800.00E-18 Rs=1m Cjo=10p)

.model qx NPN(Is=800.0E-18 Bf=93.75)

.ends

.TRAN 50US 4MS

.PROBE

.END

## RESULT:

CONCLUSION: We are able to visualise the expected output of Differentiator circuit as given in theory.

EXPERIMENT NO.10

AIM: Simulate and study basic AND, OR, NOT, NOR, NAND, EX-OR gates using PSPICE windows.

CIRCUIT DIAGRAM:

AND GATE

CLK

V

U1

1

3

2

AND2

OFFTIME = .2uSDSTM1 ONTIME = .5uS

DELAY = 0

STARTVAL = 0

OPPVAL = 1

OFFTIME = .5uSDSTM2

## NAND GATE

OFFTIME = .2uSDSTM3 ONTIME = .5uS CLK DELAY = 0

STARTVAL = 0

OPPVAL = 1

V

U5

1

3

2

NAN2	V

ONTIME = .2uS DELAY = 0

STARTVAL = 0

OPPVAL = 1

## OR GATE

CLK

V

OFFTIME = .3uSDSTM4 ONTIME = .1uS CLK DELAY = 0

STARTVAL = 0

OPPVAL = 1

## NOR GATE

OFFTIME = .2uSDSTM3 ONTIME = .5uS CLK

DELAY = 0

STARTVAL = 0

OPPVAL = 1

U6

1

3

2

OFFTIME = .2uSDSTM3 ONTIME = .5uS CLK

DELAY = 0

STARTVAL = 0

OPPVAL = 1

U4

OR2	V	1 	3

2

OFFTIME = .3uSDSTM4 ONTIME = .1uS CLK

DELAY = 0

STARTVAL = 0

OPPVAL = 1

## NOT GATE

OFFTIME = .3uSDSTM4 ONTIME = .1uS CLK

DELAY = 0

STARTVAL = 0

OPPVAL = 1

NOR2	V

CLK

V

1

OFFTIME = .2uSDSTM3 ONTIME = .5uS

DELAY = 0

STARTVAL = 0

OPPVAL = 1

## NOR GATE

2

INV

V

U9

OFFTIME = .2uSDSTM3 ONTIME = .5uS CLK

DELAY = 0

STARTVAL = 0

OPPVAL = 1

U7

1

3

2

XOR	V

OFFTIME = .3uSDSTM4 ONTIME = .1uS CLK

DELAY  = 0

STARTVAL = 0

OPPVAL = 1

## OUTPUT:

- AND GATE

- OR GATE

- NOT GATE

- NAND GATE

- NOR GATE

- X-OR GATE